Lab Assignment

- Write a program with indexed addressing that calculates the sum of all the gaps between array elements. The array elements are word type, sequenced in non-decreasing order. Use the value array {0,2,5,9,10}. These values has a gaps of 2,3,4, and 1 which the total of 10.

# Example Output:

# section .data

# ; The array elements are word type (16-bit)

# array dw 0, 2, 5, 9, 10

# array_len equ ($ - array) / 2

# msg db "Sum of gaps: "

# msg_len equ $ - msg

# newline db 10

# section .bss

# char_buffer resb 2  ; Buffer to hold the 2-digit string output

# section .text

# global _start

# _start:

# mov rsi, array      ; Load address of the array into rsi

# mov rcx, array_len  ; Loop counter

# sub rcx, 1          ; We compare pairs, so we need length - 1 iterations

# xor rax, rax        ; Clear rax (will hold the running sum)

# xor rbx, rbx        ; Clear rbx

# xor rdx, rdx        ; Clear rdx

# calc_loop:

# movzx bx, word [rsi]       ; Load current element (zero-extended to 64-bit)

# movzx dx, word [rsi + 2]   ; Load next element (zero-extended to 64-bit)

# sub dx, bx                 ; Calculate gap: next - current

# add eax, edx               ; Add the gap to our running sum in eax

# add rsi, 2                 ; Move pointer 2 bytes forward (since it is a WORD array)

# loop calc_loop             ; Decrement rcx, jump if not zero

# ; At this point, eax contains the total sum (10).

# ; We must convert this integer into ASCII characters ('1' and '0') to print it.

# mov rdi, char_buffer

# mov rcx, 10

# xor rdx, rdx

# div rcx         ; Divide eax by 10. Quotient goes to eax, remainder to edx.

# add al, '0'     ; Convert quotient (tens digit) to ASCII

# mov [rdi], al   ; Store in buffer

# add dl, '0'     ; Convert remainder (ones digit) to ASCII

# mov [rdi+1], dl ; Store in buffer

# ; Syscall: Write "Sum of gaps: "

# mov rax, 1

# mov rdi, 1

# mov rsi, msg

# mov rdx, msg_len

# syscall

# ; Syscall: Write the calculated ASCII number

# mov rax, 1

# mov rdi, 1

# mov rsi, char_buffer

# mov rdx, 2

# syscall

# ; Syscall: Write newline

# mov rax, 1

# mov rdi, 1

# mov rsi, newline

# mov rdx, 1

# syscall

# ; Syscall: Exit

# mov rax, 60

# xor rdi, rdi

# syscall

# Output

- Write an assembly code to generate a sequence of number when a number is initialized. Follow the below instruction:

  - Initialize a number which is 8.

  - Use all available register to perform an instruction that can generate a sequence of number 1,2,3,4,5,6,7,8,. The output should be like this:

1 2 3 4 5 6 7 8

2 3 4 5 6 7 8

3 4 5 6 7 8

4 5 6 7 8

5 6 7 8

6 7 8

7 8

8

# Example Output:

# section .data

# newline db 10

# section .bss

# char_buffer resb 1

# section .text

# global _start

# _start:

# mov r8, 1       ; Outer loop counter (represents the starting digit of the row)

# mov r9, 8       ; The initialized maximum number

# outer_loop:

# cmp r8, r9      ; Check if we have printed all rows

# jg end_prog

# mov r10, r8     ; Inner loop counter (starts at the current row's starting digit)

# inner_loop:

# cmp r10, r9     ; Check if we have reached 8 in the current row

# jg print_newline

# ; Convert integer to ASCII and store in buffer

# mov al, r10b

# add al, '0'

# mov [char_buffer], al

# ; Syscall: Write the character

# ; Note: We must preserve our loop registers (r8, r9, r10) by pushing them to the stack

# ; because syscalls can alter standard registers.

# mov rax, 1

# mov rdi, 1

# mov rsi, char_buffer

# mov rdx, 1

# push r8

# push r9

# push r10

# syscall

# pop r10

# pop r9

# pop r8

# inc r10         ; Increment inner loop

# jmp inner_loop

# print_newline:

# ; Syscall: Write newline at the end of the row

# mov rax, 1

# mov rdi, 1

# mov rsi, newline

# mov rdx, 1

# push r8

# push r9

# syscall

# pop r9

# pop r8

# inc r8          ; Increment outer loop to start the next row

# jmp outer_loop

# end_prog:

# ; Syscall: Exit cleanly

# mov rax, 60

# xor rdi, rdi

# syscall

Output

3. 		Write a program that prompts the user for three 32-bit integers stores them in an array, calculates the sum of the array, and displays the sum on the screen.

# Example Output:

section .data

prompt db "Enter 32-bit integer: ", 0

prompt_len equ $ - prompt

result_msg db "The sum of 32-bit integers is: +", 0

result_len equ $ - result_msg

newline db 10

section .bss

input_buffer resb 32     ; Buffer for raw keyboard input

array resq 3             ; Array to store 3 64-bit integers (quadwords)

output_buffer resb 32    ; Buffer for integer-to-string conversion

section .text

global _start

_start:

mov r12, 0               ; r12 will be our array index counter (0 to 2)

input_loop:

cmp r12, 3               ; Have we collected 3 integers?

je calculate_sum

; --- 1. Print Prompt ---

mov rax, 1

mov rdi, 1

mov rsi, prompt

mov rdx, prompt_len

syscall

; --- 2. Read Input ---

mov rax, 0               ; sys_read

mov rdi, 0               ; standard input

mov rsi, input_buffer

mov rdx, 32              ; read up to 32 bytes

syscall

; --- 3. Convert ASCII to Integer (atoi) ---

mov rsi, input_buffer    ; pointer to the string

xor rax, rax             ; rax will hold the resulting integer

xor rcx, rcx             ; clear rcx for byte extraction

atoi_loop:

mov cl, byte [rsi]       ; load next byte

cmp cl, 10               ; check for newline (Enter key)

je store_in_array

cmp cl, 0                ; check for null terminator

je store_in_array

sub cl, '0'              ; convert ASCII character to raw number (e.g., '5' - '0' = 5)

imul rax, 10             ; multiply current accumulator by 10

add rax, rcx             ; add the new digit

inc rsi                  ; move to next character

jmp atoi_loop

store_in_array:

; Store the parsed binary integer into the array

; r12 is the index (0, 1, 2). Multiply by 8 because each integer is 8 bytes (64-bit).

mov [array + r12*8], rax

inc r12

jmp input_loop

calculate_sum:

; --- 4. Sum the Array ---

xor rax, rax             ; rax will hold the sum

mov r12, 0               ; reset index

sum_loop:

cmp r12, 3

je print_result_prefix

add rax, [array + r12*8] ; add array element to sum

inc r12

jmp sum_loop

print_result_prefix:

; Save the sum (rax) to stack before syscalls destroy it

push rax

; Print "The sum is: +"

mov rax, 1

mov rdi, 1

mov rsi, result_msg

mov rdx, result_len

syscall

pop rax                  ; Restore the sum into rax

; --- 5. Convert Integer back to ASCII String (itoa) ---

mov rsi, output_buffer   ; pointer to end of buffer (we build string backwards)

add rsi, 31              ; start near the end of the 32-byte buffer

mov byte [rsi], 0        ; null-terminate the string

mov rcx, 10              ; divisor for base 10

itoa_loop:

xor rdx, rdx             ; clear rdx before division

div rcx                  ; rax / 10. Quotient in rax, remainder in rdx.

add dl, '0'              ; convert remainder to ASCII character

dec rsi                  ; move pointer back one byte

mov [rsi], dl            ; store character

test rax, rax            ; is quotient zero?

jnz itoa_loop            ; if not, keep dividing

; --- 6. Print the Converted String ---

; rsi now points to the beginning of our generated ASCII string

mov rdi, rsi             ; save string pointer

; calculate length of generated string

mov r8, output_buffer

add r8, 31

sub r8, rsi              ; r8 now contains string length

mov rax, 1

mov rdx, r8              ; length

mov rsi, rdi             ; pointer

mov rdi, 1               ; stdout

syscall

; --- 7. Print Newline and Exit ---

mov rax, 1

mov rdi, 1

mov rsi, newline

mov rdx, 1

syscall

mov rax, 60

xor rdi, rdi

syscall

Output

4. Create a procedure that receives an integer value between 0 and 100. Then, display a single capital letter on the screen. The letter returned by the procedure should be according to the following ranges.

90 to 100	A

80 to 89	B

70 to 79	C

60 to 69	D

0 to 59	F

# Example Output:

# section .data

# prompt db "Enter mark (0-100): ", 0

# prompt_len equ $ - prompt

# result_msg db "Grade: ", 0

# result_len equ $ - result_msg

# newline db 10

# ; Define grade characters

# grade_A db "A", 10

# grade_B db "B", 10

# grade_C db "C", 10

# grade_D db "D", 10

# grade_F db "F", 10

# section .bss

# input_buffer resb 8

# section .text

# global _start

# _start:

# ; --- 1. Print Prompt ---

# mov rax, 1

# mov rdi, 1

# mov rsi, prompt

# mov rdx, prompt_len

# syscall

# ; --- 2. Read Input ---

# mov rax, 0

# mov rdi, 0

# mov rsi, input_buffer

# mov rdx, 8

# syscall

# ; --- 3. Convert ASCII to Integer ---

# mov rsi, input_buffer

# xor rax, rax

# xor rcx, rcx

# parse_loop:

# mov cl, byte [rsi]

# cmp cl, 10

# je evaluate_grade

# cmp cl, 0

# je evaluate_grade

# sub cl, '0'

# imul rax, 10

# add rax, rcx

# inc rsi

# jmp parse_loop

# evaluate_grade:

# ; --- 4. Print Result Prefix ("Grade: ") ---

# push rax                 ; save parsed integer score

# mov rax, 1

# mov rdi, 1

# mov rsi, result_msg

# mov rdx, result_len

# syscall

# pop rax                  ; restore score into rax

# ; --- 5. Comparison Logic ---

# cmp rax, 90

# jge assign_A

# cmp rax, 80

# jge assign_B

# cmp rax, 70

# jge assign_C

# cmp rax, 60

# jge assign_D

# jmp assign_F             ; If it's less than 60, it's an F

# assign_A:

# mov rsi, grade_A

# jmp print_grade

# assign_B:

# mov rsi, grade_B

# jmp print_grade

# assign_C:

# mov rsi, grade_C

# jmp print_grade

# assign_D:

# mov rsi, grade_D

# jmp print_grade

# assign_F:

# mov rsi, grade_F

# jmp print_grade

# print_grade:

# ; --- 6. Print the Grade Character and Exit ---

# ; rsi already points to the correct character (which includes a newline)

# mov rax, 1

# mov rdi, 1

# mov rdx, 2               ; 1 byte for char, 1 byte for newline

# syscall

# mov rax, 60

# xor rdi, rdi

# syscall